| CONTACT US: ### Dallas 909 Regal Row Dallas, Texas 75247 214-637-3131 ### Grand Prairie 2725 S. State Highway 360 Grand Prairie, Texas 75052 469-263-1111 ### Richardson 124 E. Arapaho Richardson, Texas 75081 214-239-3131 ### Lewisville 201 Marina Vista Dr. Suite 100 Lewisville, Texas 75056 469-888-4055. |  |  | # Wood Door # Care, Maintenance, and Warranty. |
| --- | --- | --- | --- |
| # Door Basics - Allow doors to acclimate to local conditions for at least 24 hours prior to finishing - Remove all hardware including door sweeps and weatherstripping - Inspect all glass or glass inserts for excess sealant and carefully remove if any is present - Mask off all glass prior to finishing - Finish with coatings that are resistant to water, wear, heat, solvents and chemicals. Without these characteristics, the finished product may deteriorate, compromising the integrity of the product - Use a topcoat containing a UV inhibitor. Ensure that all finishing products are made for exterior surface use. This is recommended for interior units as well as they may still be exposed to sunlight. This will provide extra protection and increased durability - Finishing is recommended in a dust free area away from direct sunlight - Doors must be cleaned and dry before finishing - Finishing topcoats are recommended to be applied in several thin layers versus fewer heavier coats. If it is a wood door this will help to ensure the grain and texture is maintained - Follow all manufacturer directions on finishing products - Make sure to finish all six sides of the door slab including all hardware cutouts, as well as the frame on all sides, and any trim that will be used. - All door sweeps should be sealed on both ends after attached at the end of finishing. |  |  |  |  |  |  |  |
| # Wood Doors Painting a Wood Door 1. Clean the door with a clean, lint-free cloth, removing all dust and foreign debris. Allow the door to dry thoroughly. 2. Seal an un-primed door slab with a good quality water-based primer. 3. Paint all six sides of the door, jamb and molding with a good-quality exterior water-based 100% acrylic latex or solvent-based paint. Spray applications work best. To simulate a traditional stile and rail door look when using a paintbrush, paint the panels first, then across all the rails from one edge of the door to the other. Then paint the stiles from the top of the door to the bottom. The door can also be rolled for a stipple look. For best results, apply multiple light coats Staining a Wood Door 1. Clean the door with a clean, lint-free cloth, removing all dust and foreign debris. Allow the door to dry thoroughly. 2. For un-primed units to be stained, we recommend applying a pre-stain wood conditioner prior to staining to promote a uniform appearance and avoid sharp color contrasts or a blotchy appearance. 3. To add color after pre-stain conditioner, use an exterior oil-based stain for the next coat to all six sides of the door. 4. Topcoats may be an oil-based or water-based clear polyurethane finish. A marine-grade varnish is recommended on all exterior doors and should be applied to all six sides of the door. |  |  |  |  |  | Add a picture caption here.. |

Warranty

Door Warranty

All  doors are warranted for TWO (2) years from the date of shipment to be of good material and workmanship, and to be free from defects at the time of shipment that would render them unfit for the purpose for which each door is recommended. Should the warranted door be found to contain defects as stated above, Seconds and Surplus will at its option, either (1) repair any door without charge, or (2) replace any door without, or (3) refund the price received for the door.  Under the terms of this warranty, our obligation is to replace the defective product. No labor, installation, finishing and/or consequential damages will be covered. 1. Doors must be installed providing protection from exposure; typically, with a standard overhang extending outward and to the sides by a distance equal to one-half its height from the bottom of the door up to the overhang to be covered by this warranty. 2. Any deterioration caused by failure to protect and seal all exposed surfaces and edges of the door either prior to or immediately after hanging are not covered by this warranty. For continued protection, the finish needs to be maintained annually. 3. Seconds and Surplus may require that an inspection be made by one of our employees and/or partners prior to adjustment or replacement. 4. Variations in color, texture, grain or the general surface appearance of wood naturally occurring and not considered defects. 5. Any condition or problem caused by failure to follow proper handling, finishing and installation are not covered by this warranty, such as installing in full exposure to sun or sprinklers and behind storm doors. 6. The appearance of field-finished doors is not covered by this warranty in any event.

Glass Warranty

Seconds and Surplus warrants that all the glass (clear insulated and decorative insulated) shall be free from premature failure or permanent material obstruction of vision due to a failure of the glass seal for FIVE (5) years from the date of Purchase. If Seconds and Surplus is given notice of a glass defect occurring within FIVE (5) years of the date of purchase, Seconds and Surplus shall, at its option either (1) repair or replace the defective glass or (2) refund the original purchase price. Glass breakage isn’t warrantable, including decorative pieces between/inside the glass panel. This warranty does not apply if the failure of the seal is caused by an Act of God, damage as a result of exposure to corrosive fumes or condensates, damage resulting from excessive stress from movement of the structure, glass distortion or breakage, mishandling or improper installation or by any other cause whatsoever not within our exclusive control, and shall not apply unless the failure occurs and claim is delivered to us within ten (10) years after the date of manufacture of the insulated glass panel. Under the terms of this warranty, our obligation is to replace the defective glass. No labor, installation and/or consequential damages will be covered. We may require that an inspection be made by one of our employees and/or partners prior to adjustment or replacement.

# Warranty Exclusions

- Lack of adequate overhang (must be half the height of the door)

- Lack of proper initial finish by the customer

- Lack of regular finish upkeep and maintenance

- Staining or painting your door a very dark or black color

- Furthermore, installing a storm door over your wood door will void the warranty

Overhang Requirements

A wood entry door can last a lifetime when installed in an adequate opening and properly maintained with regular frequency.  All wood doors must be installed beneath an overhang that extends outward at least a distance equal to one-half the distance from the bottom of the door up to the overhang (see diagram to below) as a condition of the warranty.